Tutorial: Timer Interrupt & PWM Output

I. Overview

In this lab, you will learn how to set up MCU Timers for Timer Interrupt and PWM output.

Objectives of this lab are learning how to

- Configure registers of Timers(TIMx)

- Generate Timer Interrupt

- Generate PWM signals

Preparation:

- You need to study the following registers: Timer(Advanced and General Purpose, TIMx) in ‘STM Reference Manual’

- User defined APIs for GPIO and External Interrupt control

II. Tutorial – Timer Interrupt / No Output

A.  Timer Registers: TIMx

- List TIMx registers for this LAB

| Class | Register Name | Description |
| --- | --- | --- |
| TIMx | TIMx _CR1 | TIMx control register 1 |
|  | TIMx _PSC | TIMx prescaler register |
|  | TIMx _ARR | TIMx auto-reload register |
|  | TIMx_DIER | TIMx DMA Interrupt Enable register |
|  | TIMx _BDTR | TIM1(only) break and dead-time register |

- Block Diagram for General Purpose Timer

- Block Diagram for General Purpose Timer

- TIM_TypeDef :  <stm32f411xe.h>

B. Register Initialization Process

Process of Timer(TIMx) register initiation: Timer Interrupt of Over/Underflow of Counter

System Clock setting - RCC setting (HSE/PLL) Timer setting 1. Enable Timer Peripheral Clock (RCC_APB1ENR) 2. Set Counting Direction (TIMx_CR1DIR) 3. Set Timer Clock Pre-scaler value (TIMx_PSCPSC[15:0]) 4. Set Auto-reload value (TIMx_ARR->ARR) 5. Enable Timer DMA/Interrupt. (TIMx_DIERUIE) 6. Enable counter (TIMx_CR1CEN) NVIC setting - Enable TIMx Interrupt: NVIC_EnableIRQ(TIMx_IRQn) - Set interrupt Priority NVIC_SetPriority(TIMx_IRQn,2)

C. Exercise

- Timer interrupt for every 1 sec.

- System CLK is PLL 84MHz for STM32F411RE

- Use Counter of TIM2: Up-counting, Timer2_CLK = 100 kHz,  COUNT_CLK=1 kHz

- Fill in the table

| Port/Pin | Description | Register setting |
| --- | --- | --- |
| RCC | PLL Initialization | RCC_PLL_init(); EC_SYS_CLK =EC_PLL= 84,000,000Hz |
|  | Enable Timer Peripheral Clock: TIM2 | RCC->APB1ENR ¦=1≪0 |
| TIM2 | TIM2 counting direction: DIR0 | TIM2->CR1 & = ~ (1≪4) |
|  | Set Timer Clock Pre-scaler value 84MHz To 100kHz | TIM2->PSC = 840-1 |
|  | Set Auto-reload value: With 100kHz, counting of 1kHz | TIM2->ARR = 100-1 |
|  | Enable Timer DMA/Interrupt | TIM2->DIER ¦= 1<<0 // TIM_DIER_UIE |
|  | Enable Counter | TIM2->CR1 ¦= 1<<0 // TIM_CR1_CEN |
| NVIC | Set TIM2_IRQn with Priority 2, and enable | NVIC_SetPriority(TIM2_IRQn,2); NVIC_EnableIRQ(TIM2_IRQn); |

- Firmware Programming

- Create a new project and name the project as ‘Tutorial_TIMER_Interrupt’.

- Create a new item called ‘main.c’

- Then, using timer interrupt (  “ void TIM2_IRQHandler(void)” ), make LED LD2 turn On and Off in every 1 sec: ON for 1sec, OFF for 1 sec etc.

III. Tutorial – PWM Output

A.  Timer Registers: TIMx

- List TIMx registers: Timer Setting

| Class | Register Name | Description |
| --- | --- | --- |
| TIMx | TIMx _CR1 | TIMx control register 1 |
|  | TIMx _PSC | TIMx prescaler register |
|  | TIMx _ARR | TIMx auto-reload register |
|  | TIMx_DIER | TIMx DMA Interrupt Enable register |
|  | TIMx _BDTR | TIM1(only) break and dead-time register |

- List TIMx registers: PWM output

| Class | Register Name | Description |
| --- | --- | --- |
| TIMx | TIMx _CCMRn | TIMx capture/compare mode register for nth channel |
|  | TIMx _CCRn | TIMx capture/compare register for nth channel |
|  | TIMx _CCER | TIMx capture/compare output enable register |
|  | TIMx _BDTR | TIM1(only) break and dead-time register |

- Block Diagram for General Purpose Timer

B. Register Initialization Process

Process of Timer(TIMx) PWM output register initiation

GPIO Pin setting 1. Set RCC for GPIO 2. AF(TIMx) mode selection for Pin_y in GPIOx Timer setting: 1. Enable Timer peripheral Clock (RCC_APB1ENR) 2. Set Counting direction (TIMx_CR1DIR) 3. Set Timer clock Pre-scaler value (TIMx_PSCPSC[15:0]) 4. Set Auto-reload value (TIMx_ARR->ARR) 5. Enable Counter (TIMx_CR1CEN) PWM Out setting: 1. Set PWM Output mode (TIMx_CCMR OCnM) 2. Set CompareCapture value (TIMx_CCRn->CCR) 3. Select Output Polarity (TIMx_CCERCCyP) 4. Enable CompareCaptureOutput (TIMx_CCERCCyE)

C. Exercise

PWM output of period 1kHz with duty ratio 50%. PWM Out on pin PA_5 (TIM2_CH1)

- Timer Clock

- System CLK is PLL 84MHz for STM32F411RE

- Use Counter of TIM2: Up-counting, Timer2_CLK = 100 kHz,  COUNT_CLK=1 kHz

- PWM output: TIM2_CH1 / PA_5

- There are several pins for TIM2CH1. We will use GPIO Pin5 (LD2) for tutorial

- Set the GPIO pin as Alternate function (AF) for TIM2 / No pull-up & No pull-down / High speed / Push-Pull

- PWM mode 1, Duty ratio 50%

- Fill in the table

| Port/Pin | Description | Register setting |
| --- | --- | --- |
| RCC | PLL Initialization | RCC_PLL_init(); EC_SYS_CLK =EC_PLL= 84,000,000 |
|  | Enable Timer Peripheral Clock: TIM2 | RCC->APB1ENR ¦=1≪0 |
| TIM2 (Timer setting) | TIM2 counting direction: DIR0 | TIM2->CR1 & = ~ (1≪0) |
|  | Set Timer Clock Pre-scaler value 84MHz To 100kHz | TIM2->PSC = 840-1 |
|  | Set Auto-reload value: With 100kHz, counting of 1kHz | TIM2->ARR = 100-1 |
|  | Enable Timer DMA/Interrupt | TIM2->DIER ¦= 1<<0 |
|  | Enable Counter | TIM2->CR1 ¦= 1<<0 |
| GPIOA | Set GPIOA pin 5 as Alternate Function mode: 10 | GPIOA->MODER &=~ 3<<Y*5 GPIOA->MODER ¦= 2<<Y*5 |
|  | GPIOA Alternate Function Selection: AF1(TIM2/TMI5) AFRL_5[3:0]=0001 | GPIOA->AFR[0] = (pin%4)<<(4*LED_PIN); |
|  | AF Output as No-PUPD, Push-Pull,Very High Speed. | // write your HAL API __________________ __________________ |
| TIM2 (PWM setting) | Enable auto-reload preload | TIM2_CR1________________________ |
|  | Output compare mode as PWM mode1 | TIM2_CCMRn________________________ |
|  | Set CCR value for 50% duty ratio: ARR/2 | TIM2_CCR1________________________ |
|  | Output as active high | TIM2_CCER________________________ |
|  | Enable Compare and Capture output | TIM2_CCER________________________ |

- Firmware Programming

- Create a new project and name the project as ‘Tutorial_TIMER_PWMout’.

- Give PWM output of 0%, 50%, 100% of 1kHz PWM

- Check the brightness of LD2 (PA5) for each different Duty ratio

- Change to 50% Duty ratio of 10kHz frequency PWM

Appendix

- Pin Configuration of NUCLE-F411RE

- Timer GPIO pinout for STM32f411